[image: image1.png]


Online Ground School
Master Lesson Plan

1.06 Air Law


1.06 Air Law

ATC, Flight Planning, and Rules of the Air

Class Length: One 40 minute period

References: 
1. TC AIM

2. CARs

3. FTGU Pages 108-113, 

Preparation: 
Familiarize yourself with the lesson material and the PowerPoint provided.


Introduction: 

Introduce yourself briefly. This lesson covers air traffic control, flight planning, and rules of the air.


Motivation:
Information in this lesson is examinable material for either the Glider or Power Pilot Scholarship entrance exam. Understanding this lesson will help you prepare for either of these exams, and should be supplemented by your own self-study program. This material is important because all pilots are responsible for applying this knowledge when flying.

MTPs:  

1. Air Traffic Control

2. Clearances and Instructions

3. Flight Plans and Itineraries

4. Right of Way

5. Rules of the Air


Body:
1. Air Traffic Control
a. area control centre established to provide ATC service to:

i. IFR and controlled VFR (CVFR) flights

ii. IFR flights and CVFR flights operating within a control area

iii. airport traffic as the circumstances require

b. Job is

i. To prevent collisions between

1. Aircraft

2. aircraft and obstructions

3. aircraft and vehicles on the manoeuvring area

ii. as well as expediting and maintaining an orderly flow of traffic
2. Clearances and Instructions
a. ATC Clearance

i. Authorization by an ATC unit for an a/c to proceed within controlled airspace under specified conditions

ii. Once accepted, it must be executed unless you make alternate arrangements

b. ATC Instruction
i. A directive issued by an ATC unit for ATC purposes
ii. Must be accepted unless it is manifestly unsafe
3. Flight Plans and Itineraries
a. Flight Plan

i. Filed with an ATC or FIC 

ii. Provides cross country route information as well as A/C specific details that would aid in search and rescue efforts

iii. Search and rescue notified after 1 hour overdue

b. Flight Itinerary
i. Less formal than the Flight Plan
ii. Filed with a responsible person

iii. Search and rescue notified after 24hrs overdue
c. Arrival Reports
i. Contents of an arrival report for a flight plan or flight itinerary shall include:

1. type of flight plan or flight itinerary

2. the departure aerodrome

3. the arrival aerodrome

4. the date and time of arrival
3.5
Confirmation Questions

a. What is the difference between and ATC Instruction and a clearance?

a. Instruction

i. Must be complied with unless it is manifestly unsafe
b. Clearance

i. Must be complied with after accepted unless other arrangements are made
b. Fill in the blanks:

a. A flight itinerary is filed with a responsible person

b. A flight plan is filed with an ATC or an FIC
4. Right of Way
a. If there is risk of collision, the pilot in command (PIC) with right of way must take action to avoid it

b. Aircraft with emergency has the right of way

c. Order of priority:

i. Powered aircraft give way to airships, gliders and balloons; 

ii. Airships give way to gliders and balloons; 

iii. Gliders give way to balloons; and 

iv.  Powered aircraft give way to aircraft that are towing objects or carrying a slung load
d. Aircraft converging at the same altitude

i. Aircraft with the other on its right gives way

e. Aircraft approaching head on

i. Both divert course to the right

f. Aircraft both going in to land

i. Faster gives way to slower

ii. Higher gives way to lower

1. Lower aircraft shall not overtake or cut in front of higher aircraft in the final stages of approach

4.5
Confirmation Questions

a. Who has right of way when 2 aircraft are at the same altitude?

a. The aircraft on the right
b. If heading straight on to another aircraft, which way shall you both turn?

a. To the right
c. When landing, which aircraft has the right of way?

a. The lower aircraft, or the slower aircraft
5. Rules of the Air
a. Aircraft overtaking another aircraft shall pass on the right (unless ridge soaring: on the inside)

i. An aircraft in flight or manoeuvring on the surface must give way to an aircraft that is landing

ii. Do not pass over or under, or cross ahead of, another aircraft if you are not far enough away to not create a risk of collision

b. No person shall create a hazard to persons or property on the surface by dropping an object from an aircraft in flight

c. Minimum altitudes

i. Over built up areas, an aircraft must remain 1000’ above and 2000’ from an obstacle (except for take off and landing)

ii. Over rural areas, an aircraft must remain 500’ from an obstacle (except for take off and landing)

iii. Exceptions: 

1. for the purpose of a police operation

2. for the purpose of saving human life

3. for the purpose of fighting forest fires

4. for the purpose of flight training when a qualified instructor is onboard

d. No person shall operate an aircraft in formation with other aircraft except by pre-arrangement between the PICs; or where the flight is conducted within a control zone, between PICs and the appropriate ATC unit

e. Aerobatics shall not be conducted:

i. Over a built up area or an open assembly of people

ii. In controlled airspace except with a special flight operations certificate 

iii. When visibility is less than 3 miles

iv. Below 2000’ AGL except in accordance with special flight operations certificate

v. With passengers on board unless the PIC has:

1. 10 hours of aerobatic instruction or 20 hours conducting aerobatics

2. At least one hour of conducting aerobatic manoeuvres in the last 6 months



Confirmation:

1. Fill in the missing distances
a. 1000’, 2000’, 500’, 500’

2. What do you need to do before you can fly in formation?
a. Prearrange it with the other pilot

b. Obtain ATC permission if you will be flying in a control zone

3. When can you go below the minimum altitudes?

a. While taking off or landing

f. For the purpose of a police operation

g. For the purpose of saving human life

h. For the purpose of fighting forest fires

b. For the purpose of flight training when a qualified instructor
4. What requirements must you meet to fly aerobatics

a. Aerobatics shall not be conducted:

i. Over a built up area or an open assembly of people

ii. In controlled airspace except with a special flight operations certificate 

iii. When visibility is less than 3 miles

iv. Below 2000’ AGL except in accordance with special flight operations certificate

v. With passengers on board unless the PIC has:

1. 10 hours of aerobatic instruction or 20 hours conducting aerobatics

2. At least one hour of conducting aerobatic manoeuvres in the last 6 months


Conclusion:
This lesson covered air traffic control, flight planning, and rules of the air. This material is important because all pilots are responsible for applying this knowledge when flying. Understanding this lesson will help you prepare for a flying scholarship entrance exam, and should be supplemented by your own self-study program. If you have any questions, you can direct them to emailonlinegroundschool@gmail.com.

















































































1.06
5/5
Updated on 6 September 2013

